Error Calculation & Measurement Uncertainty

Method Dossier | Basic Level

1. Method Description

Systematic errors (bias) vs random errors (noise). Gaussian error propagation: sigma_Y = sqrt(sum(df/dXi)^2 * sigma_Xi^2). For addition: add squares. For multiplication: add relative squares.

2. Examples

Physics: g = 2h/t^2, sigma_g/g = sqrt(0.005^2 + (2*0.01)^2) = 2.1%

Medicine: Blood pressure 138/142/134 mmHg. Mean=138, SD=4.0, 95% CI = 138+/-9.9 mmHg (n=3 is insufficient).

Sports: Jump force 5 trials, SD=120N, relative error = 2.2% -> improve to 1.6% with 10 trials.

3. Assumptions

- Random errors normally distributed

- Input errors independent (or include covariance)

- Systematic errors corrected or reported separately

4. Reporting

Value +/- uncertainty (unit) at k=2 (95% CI). Relative uncertainty in %.

5. Code Examples

Python:

import numpy as np; from scipy.stats import t

m,s = np.mean(data), np.std(data, ddof=1)

ci = t.interval(0.95, df=len(data)-1, loc=m, scale=s/np.sqrt(len(data)))

6. Extension

GUM standard: Type A (statistical) + Type B (other) uncertainty. Monte Carlo for nonlinear functions.

7. Exercises

1. Reaction times: 220,245,235,210,240 ms. Mean, SD, SE, 95% CI (t=2.776, df=4). How many trials for SE<5ms?

2. Density rho = m/V from m=100+/-1g, V=50+/-2cm^3. rho with error propagation. Relative error?